August 2005

This file shows the bytes exchanged by the telnet client and the telnet

server (telnetd) during connection and login.

Tops is used between client and server to capture the hex dump of bytes

shown below.  RFC 854 defines the meaning of the bytes (see Reference

1).

telnet client <---socket 6---> tops <---socket 7---> telnetd server

References:

1. Postel, J., and Reynolds, J. K., "Telnet Protocol Specification,"

RFC 854, May 1983.

See:

http://www.faqs.org/rfcs/rfc854.html

http://rfc.sunsite.dk/rfc/rfc854.html

2. Telnet source (netkit-telnet-0.17.tgz):

ftp://ftp.uk.linux.org/pub/linux/Networking/netkit

The client view:

[dale@clacker] /home/dale > telnet clacker 9877

Trying 127.0.0.1...

Connected to localhost.localdomain (127.0.0.1).

Escape character is '^]'.

Red Hat Linux release 7.2 (Enigma)

Kernel 2.4.7-10 on an i686

login: dale

Password:

Last login: Wed Aug 17 09:03:05 from localhost.localdomain

You have mail.

[dale@clacker] /home/dale >

The server view (socket 6 is client, socket 7 is server):

Tops 3.0.1

Wed Aug 17 09:02:43 PDT 2005

[tops@clacker] ready > ntrace TELNETD

TELNETD: listening on port 9877

[tops@clacker] ready >

Wed Aug 17 09:02:54 PDT 2005 SERVER: connection from 127.0.0.1

[tops@clacker] ready > TELNETD_CONNECT: new client on socket 6

LOCK: locked on run level 8

Jmp table at lev 8

lev   ret  typ  Lib:lib

8     5    1     0:idle

7     2    1     0:idle

6     2    1     0:TELNETD_START

5     2    1     0:TELNETD_CONNECT

4     2    1     0:SERVE_F

3     9    1     0:console

2     2    1     0:console

1     1    2     0:DATA__

LOCK: flag to unlock received on run level 8

LOCK: unlock run level 8

TELNETD_START: telnetd is listening on port 9923

CONNECT: trying 127.0.0.1

SERVER: new client on socket 6

readn1: 12 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FD 18 FF FD 20 FF FD 23 FF FD 27 00 00 00 00  ..... ..#..'....

drainf: 12 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 12 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FD 18 FF FD 20 FF FD 23 FF FD 27 00 00 00 00  ..... ..#..'....

readn1: 12 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FB 18 FF FB 20 FF FB 23 FF FB 27 00 00 00 00  ..... ..#..'....

drainf: 12 bytes from socket 6, type -1

drainf: socket 6 set to type 3, FOREIGN

drainf: server type is 1

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 12 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FB 18 FF FB 20 FF FB 23 FF FB 27 00 00 00 00  ..... ..#..'....

readn1: 24 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FA 20 01 FF F0 FF FA 23 01 FF F0 FF FA 27 01  .. .....#.....'.

2  FF F0 FF FA 18 01 FF F0 00 00 00 00 00 00 00 00  ................

drainf: 24 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 24 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FA 20 01 FF F0 FF FA 23 01 FF F0 FF FA 27 01  .. .....#.....'.

2  FF F0 FF FA 18 01 FF F0 00 00 00 00 00 00 00 00  ................

readn1: 71 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FA 20 00 33 38 34 30 30 2C 33 38 34 30 30 FF  .. .38400,38400.

2  F0 FF FA 23 00 63 6C 61 63 6B 65 72 3A 30 2E 30  ...#.clacker:0.0

4  FF F0 FF FA 27 00 00 44 49 53 50 4C 41 59 01 63  ....'..DISPLAY.c

6  6C 61 63 6B 65 72 3A 30 2E 30 FF F0 FF FA 18 00  lacker:0.0......

8  58 54 45 52 4D FF F0 00 00 00 00 00 00 00 00 00  XTERM...........

drainf: 71 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 71 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FA 20 00 33 38 34 30 30 2C 33 38 34 30 30 FF  .. .38400,38400.

2  F0 FF FA 23 00 63 6C 61 63 6B 65 72 3A 30 2E 30  ...#.clacker:0.0

4  FF F0 FF FA 27 00 00 44 49 53 50 4C 41 59 01 63  ....'..DISPLAY.c

6  6C 61 63 6B 65 72 3A 30 2E 30 FF F0 FF FA 18 00  lacker:0.0......

8  58 54 45 52 4D FF F0 00 00 00 00 00 00 00 00 00  XTERM...........

readn1: 15 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FB 03 FF FD 01 FF FD 1F FF FB 05 FF FD 21 00  ..............!.

drainf: 15 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 15 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FB 03 FF FD 01 FF FD 1F FF FB 05 FF FD 21 00  ..............!.

readn1: 24 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FD 03 FF FC 01 FF FB 1F FF FA 1F 00 48 00 24  .............H.$

2  FF F0 FF FD 05 FF FB 21 00 00 00 00 00 00 00 00  .......!........

drainf: 24 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 24 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FD 03 FF FC 01 FF FB 1F FF FA 1F 00 48 00 24  .............H.$

2  FF F0 FF FD 05 FF FB 21 00 00 00 00 00 00 00 00  .......!........

readn1: 67 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FB 01 52 65 64 20 48 61 74 20 4C 69 6E 75 78  ...Red Hat Linux

2  20 72 65 6C 65 61 73 65 20 37 2E 32 20 28 45 6E   release 7.2 (En

4  69 67 6D 61 29 0D 0A 4B 65 72 6E 65 6C 20 32 2E  igma)..Kernel 2.

6  34 2E 37 2D 31 30 20 6F 6E 20 61 6E 20 69 36 38  4.7-10 on an i68

8  36 0D 0A 00 00 00 00 00 00 00 00 00 00 00 00 00  6...............

drainf: 67 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 67 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FB 01 52 65 64 20 48 61 74 20 4C 69 6E 75 78  ...Red Hat Linux

2  20 72 65 6C 65 61 73 65 20 37 2E 32 20 28 45 6E   release 7.2 (En

4  69 67 6D 61 29 0D 0A 4B 65 72 6E 65 6C 20 32 2E  igma)..Kernel 2.

6  34 2E 37 2D 31 30 20 6F 6E 20 61 6E 20 69 36 38  4.7-10 on an i68

8  36 0D 0A 00 00 00 00 00 00 00 00 00 00 00 00 00  6...............

readn1: 3 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FD 01 00 00 00 00 00 00 00 00 00 00 00 00 00  ................

drainf: 3 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 3 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  FF FD 01 00 00 00 00 00 00 00 00 00 00 00 00 00  ................

readn1: 7 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  6C 6F 67 69 6E 3A 20 00 00 00 00 00 00 00 00 00  login: .........

drainf: 7 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 7 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  6C 6F 67 69 6E 3A 20 00 00 00 00 00 00 00 00 00  login: .........

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  64 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  d...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  64 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  d...............

readn1: 1 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  64 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  d...............

drainf: 1 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 1 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  64 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  d...............

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  61 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  a...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  61 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  a...............

readn1: 1 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  61 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  a...............

drainf: 1 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 1 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  61 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  a...............

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  6C 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  l...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  6C 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  l...............

readn1: 1 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  6C 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  l...............

drainf: 1 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 1 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  6C 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  l...............

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  65 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  e...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  65 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  e...............

readn1: 1 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  65 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  e...............

drainf: 1 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 1 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  65 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  e...............

readn1: 2 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  0D 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  ................

drainf: 2 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 2 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  0D 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  ................

readn1: 12 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  0D 0A 50 61 73 73 77 6F 72 64 3A 20 00 00 00 00  ..Password: ....

drainf: 12 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 12 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  0D 0A 50 61 73 73 77 6F 72 64 3A 20 00 00 00 00  ..Password: ....

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  61 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  a...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  61 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  a...............

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  62 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  b...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  62 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  b...............

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  63 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  c...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  63 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  c...............

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  64 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  d...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  64 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  d...............

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  64 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  e...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  64 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  e...............

readn1: 1 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  65 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  f...............

drainf: 1 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 1 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  65 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  f...............

readn1: 2 bytes received on socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  0D 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  ................

drainf: 2 bytes from socket 6, type 3

TELNET_CLIENT: receive from client socket 6 write to telnetd socket 7

remoteputf: writing 2 bytes to socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  0D 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00  ................

readn1: 78 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  0D 0A 4C 61 73 74 20 6C 6F 67 69 6E 3A 20 57 65  ..Last login: We

2  64 20 41 75 67 20 31 37 20 30 39 3A 30 33 3A 30  d Aug 17 09:03:0

4  35 20 66 72 6F 6D 20 6C 6F 63 61 6C 68 6F 73 74  5 from localhost

6  2E 6C 6F 63 61 6C 64 6F 6D 61 69 6E 0D 0A 59 6F  .localdomain..Yo

8  75 20 68 61 76 65 20 6D 61 69 6C 2E 0D 0A 00 00  u have mail.....

drainf: 78 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 78 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  0D 0A 4C 61 73 74 20 6C 6F 67 69 6E 3A 20 57 65  ..Last login: We

2  64 20 41 75 67 20 31 37 20 30 39 3A 30 33 3A 30  d Aug 17 09:03:0

4  35 20 66 72 6F 6D 20 6C 6F 63 61 6C 68 6F 73 74  5 from localhost

6  2E 6C 6F 63 61 6C 64 6F 6D 61 69 6E 0D 0A 59 6F  .localdomain..Yo

8  75 20 68 61 76 65 20 6D 61 69 6C 2E 0D 0A 00 00  u have mail.....

readn1: 28 bytes received on socket 7

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  5B 64 61 6C 65 40 63 6C 61 63 6B 65 72 5D 20 2F  [dale@clacker] /

2  68 6F 6D 65 2F 64 61 6C 65 20 3E 20 00 00 00 00  home/dale > ....

drainf: 28 bytes from socket 7, type 1

drainf: socket 7 running ptr 3B 80000000

TELNETD_SERVER: receive from telnetd socket 7 write to client socket 6

remoteputf: writing 28 bytes to socket 6

0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F  0123456789ABCDEF

0  5B 64 61 6C 65 40 63 6C 61 63 6B 65 72 5D 20 2F  [dale@clacker] /

2  68 6F 6D 65 2F 64 61 6C 65 20 3E 20 00 00 00 00  home/dale > ....